“No Raw Edge” Pillowcase

Selvage Edge

3/4 yd. Pillow body 	 1/4 yd. Pillow Cuff 	 3 in. Accent

- Press Accent fabric in half long ways.

- Lay cuff fabric down right side facing up.

- Place Accent fabric along top edge of cuff fabric matching raw edges.

- Place body fabric right side facing down, along top edge of cuff and accent fabrics matching raw edges. Pin in place. If selvages do not match it’s OK, you can even them up later.

- Tightly roll body fabric from bottom until you can see the half of cuff fabric.

- Fold Cuff fabric up & over all the other fabrics; match the raw edge to the others, forming a tube, and pin. Double check to make sure that only the edge of the pillow body fabric is in the seam.

- Sew along the edge using ¼ in. seam allowance.

- Pull all the fabric out of the roll from one end.

To Finish with a French Seam:

- Press and fold in half wrong sides together matching cuff edge at top. Even up both side and bottom with rotary cutter or scissors making sure to cut off the selvages then pin both sides.

- Sew a 1/8 in. seam on the one side and bottom (still with wrong sides together)

- Turn Pillowcase inside out, using 1/4 in. seams sew the same side and bottom making sure the 1/8 in. seam is being sewn inside.

- Turn right side out and enjoy.

To Finish with a Simple Seam:

- Fold in half RIGHT sides together matching cuff edge at top. Even up both sideand bottom with rotary cutter making sure to cut off the selvages then pin both sides.

- Sew a ½” seam on the one side and bottom. Then zig zag raw edges to clean up the edges.

- Turn right side out and enjoy!!